AFTER LANDING

Flaps…………………………..……..………….UP

Carb Heat………………….…..….……….….Cold

Trim…………………….……..…...…...…..Neutral

Mixture…………...……………………..….....Lean

LDG/Strobe Lights…………….…….……….OFF

Taxi Clearance………………..…Obtain/ Brief

Shutdown Terminate

XPDR………......Set 1226/ALT Mode(For Leesburg)

Avionics Master…………………………….OFF

Heater/Defrost……………………...………OFF

Throttle…………………………..…..1000 RPM

Mixture……………………………….….....Cutoff

Magnetos………………………..…..….…...OFF

Switches……………………….…...…ALL OFF

Battery Master & Alternator……..……....OFF

Window…………..………………..…..…Closed

Hobbs & Tachometer……………........Record

Flight Controls……………….………....Secure

TieDown/Chocks..………………...……Secure

Clean Aircraft…………………………………

Covers…………….………………………..Install

Walkaround…………….……….....…Complete

V-Speeds

Vs0 - 41 KIAS--------Stall speed in a Landing Configuration

VS - 47 KIAS—---------Stall Speed in a Clean Configuration

VR - 55 KIAS------------------------------------------Rotation Speed

VX - 59 KIAS------------------------------------Best Angle of Climb

VY - 73 KIAS--------------------------------------Best Rate of Climb

VG - 65 KIAS(Flaps UP)/60 KIAS(Flaps Down) —----Best Glide

VFE - 85/110 KIAS-----------Maximum Flap Extension Speed

VA - 97/89/80 KIAS—------------------------Maneuvering Speed

VNO - 128 KIAS—------Maximum Structural Cruising Speed

VNE - 160 KIAS--------------------------------Never Exceed Speed

Maximum Demonstrated Crosswind—-------------15 KNOTS

PATTERN WORK

BEFORE LANDING CHECKLIST

Carb Heat…………………………………...….ON

POWER…………………….……..………As REQ

Flaps..……………………..…....(80 KIAS))AS REQ

Mixture……………..……………………..Full Rich

Trim………………………………………….…..Set

GO AROUND / MISSED APPROACH

Throttle…………………...…………...…Full FWD

Carb Heat………………...…………………...Cold

Flaps……………………………..…...…………20’

Pitch…………….…..…..….Up for Climb(55 KIAS)

Flaps…………….....10’(until Obstacles are cleared)

Pitch……………………………….………60 KIAS

STANDARDIZED SPEEDS

90 KIAS…………………………...…….….….Downwind

80 KIAS……………………...………Abeam, 1500 RPM

75 KIAS………………………………………….…..Base

65 KIAS..…...…………………………….……...On Final

65 KAIS…………..…………....…Performance Landing

60 KIAS………….…..…………Short/Soft field Landing

AFTER LANDING

Flaps……………….……..……………….…….UP

Carb Heat……………………….…….………Cold

Trim………………….………..….…………Neutral

Mixture……….…….…………………...…….Lean

Strobe Lights……...……………………….….OFF

Taxi Clearance………..……………...Obtain/Brief

BEFORE TAKEOFF

Flaps……………………………….…..….As REQ

Carb Heat……………………….…….………Cold

Engine Instruments…………………...……Check

—--------------------------Final Items—-----------------------------

Door & Window……………………...….…Secure

Lights, Camera, Action……….….XPDR, MIX, PUMP

Traffic…………………………..….………...Check

AFTER TAKEOFF

At Traffic Pattern Altitude

Airspeed……………………...……...……90 KIAS

Flaps……………………….……..……………..UP

CE-172N N566BG

BEFORE ENGINE START

Nose-Tip-Tail-Chocks Area……...…..…Clear

Preflight Inspection………………....Complete

WT&Bal, Docs, Performance………...Check

Passenger Brief…..…………...………SAFETY

Door,Emergency Exit, seats & seat Belts, Fire Extinguisher, no smoking, PIC Authority, Positive Exchange of Controls.

Seats & seat belts………...Locked/Fastened

Fuel Selector………………………….……Both

Avionics Master…………………….………OFF

Alternate Static Source………..….OFF/FWD

Carb Heat…………………….…….…….…Cold

Circuit Breakers………………...……….Check

Switches……………………….………ALL OFF

Battery Master/Alternator…..…..…………ON

Beacon…………………………..…………….ON

Nav Lights( Night REQ)......................As REQ

ENGINE START

Rested Engine………......Prime 2-6x/Locked

Throttle.………………………………...⅛” Open

Mixture………………...………...….….Full Rich

Brakes……………………..…………………Hold

Magnetos…………………….……………….ON

Prop Area………………………....”Clear Prop”

Starter…………………..………………..EngageIf the engine does not start, reference Cold Weather Operations in POH

Throttle……………...…….…Rest@1000 RPM

Oil Pressure……………………………...Check

Mixture…………………………….Lean for Taxi

AFTER START

Avionics Master………………...……………...ON

Heater/Defrost………………………….....As Req

Headsets………………………………………..On

GPS Database Currency…….…..Checked/ENT

Alternator Output…………………………...Check

Coms…………………………...….……..Set/ATIS

Altimeters….……………………..…….………Set

Clearance(IFR).........................................Obtain

GPS/NAVs…………….………………..………Set

CDI Softkey…………….….……….Set VLOC/GPS

Flight Instruments…………...……….…….Check

Transponder…….....Set Code/or receive from tower

TAXI

Do Not Ride Brakes

Sterile Cockpit……..No Non-essential Conversation

Airport Diagram………..………….…….Available

Taxi Light….…………...……...…….………….ON

Taxi Clearance…………………….....Obtain/Brief

Taxi Area…………..…….Clear Left/Right/Center

Brakes…………………….…..…………….Check

Flight Instruments………………..Check in Turns

RUN UP

May Be Completed at any Point between After Start and Before Takeoff Checklists.

Oil Temperature……….……..…...Check Normal

Fuel Gauges/ QTY……….…….…..……...Check

Fuel Selector…………………...…...……..…Both

Mixture…………………….…………..….Full Rich

Throttle………………………………....1700 RPM

Magnetos…..….....Max drop 125 RPM/Max diff. 50 RPM

Carb Heat…………………...…...(RPM Drop)Check

Engine Instruments & Ammeter……….….Check

Suction Gage…………………………….…Check

Throttle………………………………....1000 RPM

Mixture……………………….….......Lean for Taxi

Flight Controls………………………………Check

Trim………………………….…...…….Neutral/set

Engine is Warm for Takeoff when Throttle can be opened without Engine Faltering

PRE-TAKEOFF BRIEF

In the event of an Engine Failure or abnormality with enough runway remaining-

- Throttle…………………………….….Immediately Closed

- Land………………………………..…………..Immediately

- Brake………………………………….…...……….As REQ

- Stop Straight Ahead

If Not enough Runway remains to Stop-

- Throttle………………………….…….Immediately Closed

- Brake…………………………….…………….……As REQ

- Flaps…………………………………….…….……..Retract

- Mixture……………………………………….…..Idle Cutoff

- Magnetos…………………………………….…………OFF

- Battery Master & Alternator…………………….…….OFF

Avoid Obstacles

Engine Failure After Rotation with no Runway Remaining-

- Airspeed…...…………………...……...65 KIAS(Flaps Up)

60 KIAS(Flaps Down)

- Mixture………………………………….…….….Idle Cutoff

- Fuel Selector…………………….…….……Rotate to OFF

- Magnetos………………………………….……………OFF

- Flaps………………………………………………..As REQ

- Battery Master & Alternator……………….………….OFF

Land at Lowest Possible Airspeed & Avoid obstacles as Best as possible

Before  Takeoff

Engine Instruments………………..…...….Check

Flaps…………………………………........As REQ

Carburetor Heat……………….…….…..…...Cold

GPS/CDI……………………..………..….…….Set

HI/DI………………………………..…….……..Set

Departure Brief + Initial Altitude & Heading  Seat Belts & Shoulder Harness……..…....Check

-----------------------Final Items—---------------------

Door & Window……….……...……....Secure

Traffic…………………………….…...….Check

Lights/Camera/Action

Ldg-Strobe-Beacon………………………………..ON

Transponder……………………………………Set/ALT

Mixture………………………………………...Full Rich

AFTER TAKEOFF

Flaps UP……………………….….………Check

Cruise Climb………….. 75-85 KIAS/As REQ

Throttle………………….…….………Verify Full

Mixture…………….…….RICH(Lean Above 3000)

Climb at 75-85 KIAS for better visibility and cooling, Unless VY is needed for climb rate.

CRUISE

Throttle………………..…...2200-2400 RPM/Set

Mixture……………………………Lean as REQ

Trim…………………………………….………Set

Engine Instruments……..……………...Check

Maintain Situational Awareness, Monitor Frequencies, and Get ahead of the Airplane.

APPROACH

Approx. 15 NM from Airport

ATIS/AWOS……………………….……...Check

Altimeter…………………….……………..….Set

Approach briefing…………..……....Complete Fuel Selector………………………….……Both

Mixture………………………..…Slightly Enrich

Landing Light……………………….…….….ON

Airport Diagram……………….…Check/Ready

WHEN DIRECT TO IAF OR VECTORED(IFR)

Flight & NAV Instruments………....Set/Ident.

CDI Softkey……………….....VOR/LOC/GPS

BEFORE LANDING CHECKLIST

Fuel Selector……………………………….…Both

Mixture…………....................................Full Rich

Carb Heat…………………………………...….ON

Power………………….……..…………...As REQ

Flaps……………………..…………..…....As REQ

Trim………………………………….…..….....SET

GO AROUND/MISSED APPROACH

Throttle……………..……………..……..Full FWD

Carb Heat…………………………….…….…Cold

Flaps……………………………………..…...…20’

Pitch……………….……....UP for Climb(55 KIAS)

Flaps………………….……10’ (Until Positive Rate)

Pitch……………………………………….59 KIAS

Fly Assigned or Published Heading & Altitude